# LASER DENTAL INNOVATIONS

# StarLite Fiber Cleave  Instructions

# This instruction sheet will assist in learning the cleave technique for your new fiber optic. A little bit of practice and patience will result in a perfect technique.

# Cleaving Optical Fiber:

1. Lay the stripped fiber on a firm soft surface ( a post it note pad works great for this and the glue strip on the back will hold the cleaved fiber for easy disposal.)

2. Put the cleaver blade gently on the fiber surface when you are ready to cleave.

3. Point the cleaver straight up and down and use a finger on your other hand to hold the fiber. You are now ready to cleave

4. Push down on the cleaver not to hard ( you need to try this a couple times to get the right pressure) and move the blade across the fiber creating a scratch in the fiber.

Note: You are not trying to cut thru the glass just create a flaw in the surface so the glass will break evenly.

5. After you scratch the surface put the fiber between 2 fingers (be careful here) and pinch at the cleave point the fiber should break cleanly. With the laser pointing beam on observe the red spot. A correct cleave will yield a spot that is round with only minor flares. If the spot is not round or you have large flares cleave again as this will give you uniform power at the tip of the fiber.

Cleaver at 90 Degrees

Firm Soft Surface

Position the Cleaver