# LASER DENTAL INNOVATIONS

#### Exclusive Supplier of the only Bendable Cannula for Laser Dentistry that retains

#### the angle you set

# 877-753-5054 to reorder

# LD65-T or C Surgical and Hygiene Dental Laser Disposable Tip  User Instructions

Tips are packaged in a sealed plastic box

Assure paper seal is intact

Inspection of the tips after unpacking:

- Visually inspect the box for any signs of damage

- Tips comes un-sterilized and need to be cold sterilized before each use

Caution: Use Non-Flammable Sterilization Fluid only.

Tips are disposable and not FDA approved for multiple use.  After use do not re-cold sterilize or autoclave,  Please Discard !

## Loading the tips onto the LiteSaber series  handpieces

- Push large end of tip onto handpiece taper until tight

- Straighten tip slightly and feed fiber thru then let tip bend back to its natural position.

- Bend to desired angle